Pins for connectors.

## Pins for a 72-contact connector

| 72-pin connector |  |  |
| --- | --- | --- |
| pin | Side A | Side B |
| 1 |  |  |
| 2 | SPEAKER L |  |
| 3 | SPEAKER R | GND |
| 4 | HOLD1 SW |  |
| 5 | HOLD 2 SW |  |
| 6 | HOLD 3 SW |  |
| 7 | HOLD 4 SW |  |
| 8 | HOLD 5 SW |  |
| 9 | START SW |  |
| 10 | BET ONE SW |  |
| 11 | SELECT GAME SW |  |
| 12 | AUTO START |  |
| 13 |  |  |
| 14 | MAXBET SW |  |
| 15 | INFO SW |  |
| 16 | FRONT DOOR SW |  |
| 17 | BACK DOOR SW |  |
| 18 | Coin in (Pulse mode) |  |
| 19 |  |  |
| 20 | STATISTIC SW (short) |  |
| 21 | COLLECT SW |  |
| 22 |  |  |
| 23 | BILLS COUNTER | SELECT GAME LAMP |
| 24 | DROP COUNTER | WBA LED |
| 25 | REMOTE COUNTER | UPPER LAMP BLUE |
| 26 | GAMES COUNTER | UPPER LAMP WHITE |
| 27 | HANDPAY COUNTER | WIN COUNTER |
| 28 | TOKEN COUNTER | BET COUNTER |
| 29 | HOLD 1 LAMP | BET ONE LAMP |
| 30 | HOLD 2 LAMP | MAXBET LAMP |
| 31 | HOLD 3 LAMP | COLLECT LAMP |
| 32 | HOLD 4 LAMP |  |
| 33 | HOLD 5 LAMP | AUTO START LAMP |
| 34 | START LAMP | INFO LAMP |
| 35 |  |  |
| 36 | GND | GND |

For connecting the bill acceptor in the impulse regime it is necessary to apply a signal to 18А.

## Pins for a 10-contact connector

| 10-pin connector |  |  |
| --- | --- | --- |
| pin | Side A | Side B |
| 1 | GND | GND |
| 2 | GND | GND |
| 3 | +5v | +5v |
| 4 | +5v | +5v |
| 5 | +12v | +12v |
| 6 | +12v | +12v |
| 7 |  |  |
| 8 |  |  |
| 9 | GND | GND |
| 10 | GND | GND |

# Plug-in of a bill acceptor

## General information

The bill acceptor is connected to the block by means of a special interface cable according to the following table.

RS232 is used for the protocol ID003.

TTL is used for protocol SSP.

It is connected to the connector X3.

| COM Port |  |
| --- | --- |
| pin | purpose |
| 1 | RXD (TTL) |
| 2 | RXD(RS232) |
| 3 | TXD(RS232) |
| 4 | TXD (TTL) |
| 5 | ground |
| 6 | NC |
| 7 | NC |
| 8 | NC |
| 9 | NC |

Initialization:

(IMPORTANT: DO NOT ABRIDGE THE CROSSPIECE “ERASE” - THE PROGRAM WILL BE ERASED)

- while the board is switched on,  ground the lead 19А.

- reset the board having abridged the crosspiece RESET

- initialize the memory by any button

- disconnect the lead 19A from the ground

- reset the board, having abridged the crosspiece RESET, the message will appear on the screen on

the red background

- turn the key, the configuration settings will be available after that. On the first page of the settings

currency type and denomination are adjusted. On the fourth page – adjustments of the bill acceptor

channels (in accordance with a sticker on the bill acceptor).

Without initialization of the board at turning the key the following menu is available:

-ATTENDANT

-ACCOUNTING

-DIAGNOSTIC

-EVENT LOG

-GAME HISTORY

-GAME STATICTIC

At turning the key with the door opened (lead 16А to the ground) the following is available:

-GAME SETUP

-MACHINE SETUP

-CALIBRATION

In the menu MACHINE SETUP/PARAMETER SETUP the setting REMOTE INPUT

is a permission or prohibition of the key credit setting.

At the setting REMOTE INPUT switched on the key credit is set as pointed out below:

- turn the key

- press HOLD5 twice

- set the credit with the buttons HOLD3/HOLD4/HOLD5