Pins for connectors

## Pins for 72-contact  connector

| 72-pin connector |  |  |
| --- | --- | --- |
| pin | Side A | Side B |
| 1 |  |  |
| 2 | SPEAKER L |  |
| 3 | SPEAKER R | GND |
| 4 | HOLD1 SW | BILL ACCEPTOR |
| 5 | HOLD 2 SW |  |
| 6 | HOLD 3 SW |  |
| 7 | HOLD 4 SW |  |
| 8 | HOLD 5 SW |  |
| 9 | START SW |  |
| 10 | BET ONE SW |  |
| 11 | SELECT GAME SW |  |
| 12 | AUTO START |  |
| 13 | SERVIS |  |
| 14 | MAXBET SW |  |
| 15 | INFO SW |  |
| 16 | FRONT DOOR SW |  |
| 17 | BACK DOOR SW |  |
| 18 |  |  |
| 19 |  |  |
| 20 | STATISTIC SW (short) |  |
| 21 | COLLECT SW |  |
| 22 |  |  |
| 23 | BILLS COUNTER | SELECT GAME LAMP |
| 24 | DROP COUNTER | WBA LED |
| 25 | REMOTE COUNTER | UPPER LAMP BLUE |
| 26 | GAMES COUNTER | UPPER LAMP WHITE |
| 27 | HANDPAY COUNTER | WIN COUNTER |
| 28 | TOKEN COUNTER | BET COUNTER |
| 29 | HOLD 1 LAMP | BET ONE LAMP |
| 30 | HOLD 2 LAMP | MAXBET LAMP |
| 31 | HOLD 3 LAMP | COLLECT LAMP |
| 32 | HOLD 4 LAMP |  |
| 33 | HOLD 5 LAMP | AUTO START LAMP |
| 34 | START LAMP | INFO LAMP |
| 35 |  |  |
| 36 | GND | GND |

# Side А (the top) on the connector (block terminal), which is connected to the controller on 72 pin, can be determined by means of biggest quantity of appropriate wires. On the side B – lesser quantity of wires.

Pins for 10-contact connector

| 10-pin connector |  |  |
| --- | --- | --- |
| pin | Side A | Side B |
| 1 | GND | GND |
| 2 | GND | GND |
| 3 | +5v | +5v |
| 4 | +5v | +5v |
| 5 | +12v | +12v |
| 6 | +12v | +12v |
| 7 |  |  |
| 8 |  |  |
| 9 | GND | GND |
| 10 | GND | GND |

Switching on the ECS

1) 2 for 10 pin. Standard power adaptor with computer.

2) 2 for 2  pin. Power for the embedded computer system is switched from the standard power adaptor.

3) switching of the bill acceptor see the paragraph (pinout of the embedded computer system).

4) switching the monitors see the picture №1. I – medium monitor. II – low one, III-top one. (low monitor should be connected by means of adaptor dvi on vga)  monitors aiser w203

5) initialization of the machine (at the switched off  machine 1) open the door (pull the pinswitch) 2) turn the key (operator’s) and hold 3) press and hold the button Service.

4) power on the machine

Picture № 1

# Note: Connecting of the embedded computer system is analogous to connecting of Igrosoft. Positioning of the controller (of the embedded computer system) is the same as at connecting of the PCB Igrosoft.

# Connecting of the bill acceptor according to the protocol

## General information

Recommended type of the bill acceptor: MFL-0386 (validation head MFLV-2110, power interface FLP-1710). Such bill acceptor (with power interface 1710) is connected to the embedded computer system with a special interface cable, which includes integrated circuit of adapting with levels of informational signals. Bill acceptors with power interface 2810 are connected to the embedded computer system with the interface cable without the microcircuit of adapting (power interface 2810 corresponds to the standard COM-port of the embedded computer system).

## Pins for connector of the bill acceptor

### Exterior image

1

6

7

12

### Functionality of outputs

| Contact | Purpose |
| --- | --- |
| 1 | +12 V |
| 2 | M.RES |
| 3 | +12 V (INTERFACE) |
| 4 | GND |
| 5 | LED+ |
| 6 | NC |
| 7 | GND |
| 8 | TXD |
| 9 | RXD |
| 10 | NC |
| 11 | LED- |
| 12 | NC |

## Pins for COM

### Exterior image

On the connector, switched to the embedded computer system

On the embedded computer system

### Functionality of outputs

| N of output | Name | Character of signal | Signal |
| --- | --- | --- | --- |
| 1 | DCD | Input | Data carrier detect |
| 2 | RxD | Output | Transmit data |
| 3 | TxD | Input | Receive data |
| 4 | DTR | Output | Data terminal ready |
| 5 | GND | - | Ground |
| 6 | DSR | Input | Data set ready |
| 7 | RTS | Output | Request to send |
| 8 | CTS | Input | Clear to send |
| 9 | RI | Input | Ring indicator |

### Connector on the motherboard

RRXD1

DDTR#1

DDSR#1

CCTS#1

DDCD#1

TTXD1

GND

RRTS#1

RRI#1

1

2

9

### Correspondence of the connector on the motherboard to exterior connector

| Connector contact on the motherboard | Contact of the exterior connector |
| --- | --- |
| 1 (DDCD#1) | 1 (DCD) |
| 2 (RRXD1) | 2 (RxD) |
| 3 (TTXD1) | 3 (TxD) |
| 4 (DDTR#1) | 4 (DTR) |
| 5 (GND) | 5 (GND) |
| 6 (DDSR#1) | 6 (DSR) |
| 7 (RRTS#1) | 7 (RTS) |
| 8 (CCTS#1) | 8 (CTS) |
| 9 (RRI#1) | 9 (RI) |
| 10 - | - |

(All the contacts match each other)

## Connecting of the bill acceptor (power interface 1710) to the embedded computer system

The converted interface cable from the bill acceptor ICT A7 is used for connecting.

The scheme of connecting of the interface cable and power for the connector is shown below. ATTENTION! The bill acceptor connector is shown from the side of wires.

Bevel

Ground

+5 V

+12 V

Black

White

Blew

Red

Interface cable

1

ATTENTION! For some models of the interface cable power +12 V instead of +5 V is required.

## Functionality of wires of the interface cable A7

| Pin1 | GND | Blew |
| --- | --- | --- |
| Pin2 | TXD2 |  |
| Pin3 | RXD2 |  |
| Pin4 | Program |  |
| Pin5 | Reset |  |
| Pin6 | VCC | Red |
| Pin7 | RXD1 | Black |
| Pin8 | TXD1 | White |

Note: For connecting any other bill acceptor according to the protocol the same interface cable А7 can be used. For this the connector for ICT A7 is disposed, the power connector is added and an adapting pipe for a necessary bill acceptor. Connecting of the bill acceptor with impulse is possible (see. 72-pin connector).

In the menu of the embedded computer system necessary settings for the bill acceptor are chosen. For data saving it is necessary to make reset by means of switching off power.