Pins for connectors.

## Pins for a 72-contact connector

| 72-pin connector |  |  |
| --- | --- | --- |
| pin | Side A | Side B |
| 1 |  |  |
| 2 | SPEAKER L |  |
| 3 | SPEAKER R | GND |
| 4 | HOLD1 SW |  |
| 5 | HOLD 2 SW |  |
| 6 | HOLD 3 SW |  |
| 7 | HOLD 4 SW |  |
| 8 | HOLD 5 SW |  |
| 9 | START SW |  |
| 10 | BET ONE SW |  |
| 11 | SELECT GAME SW |  |
| 12 | AUTO START |  |
| 13 |  |  |
| 14 | MAXBET SW |  |
| 15 | INFO SW |  |
| 16 | FRONT DOOR SW |  |
| 17 | BACK DOOR SW |  |
| 18 | Coin in (Pulse mode) |  |
| 19 |  |  |
| 20 | STATISTIC SW (short) |  |
| 21 | COLLECT SW |  |
| 22 |  |  |
| 23 | BILLS COUNTER | SELECT GAME LAMP |
| 24 | DROP COUNTER | WBA LED |
| 25 | REMOTE COUNTER | UPPER LAMP BLUE |
| 26 | GAMES COUNTER | UPPER LAMP WHITE |
| 27 | HANDPAY COUNTER | WIN COUNTER |
| 28 | TOKEN COUNTER | BET COUNTER |
| 29 | HOLD 1 LAMP | BET ONE LAMP |
| 30 | HOLD 2 LAMP | MAXBET LAMP |
| 31 | HOLD 3 LAMP | COLLECT LAMP |
| 32 | HOLD 4 LAMP |  |
| 33 | HOLD 5 LAMP | AUTO START LAMP |
| 34 | START LAMP | INFO LAMP |
| 35 |  |  |
| 36 | GND | GND |

## Pins for a 10-contact connector

| 10-pin connector |  |  |
| --- | --- | --- |
| pin | Side A | Side B |
| 1 | GND | GND |
| 2 | GND | GND |
| 3 | +5v | +5v |
| 4 | +5v | +5v |
| 5 | +12v | +12v |
| 6 | +12v | +12v |
| 7 |  |  |
| 8 |  |  |
| 9 | GND | GND |
| 10 | GND | GND |

# Plug-in of a bill acceptor

## General information

The bill acceptor is connected to the block by means of a cable according to the following table via RS232

In the program menu you can choose one of the protocols of connecting the bill acceptor (CCNET, ID003, Mars, SSP or Puls).

For connecting the bill acceptor in the impulse regime it is necessary to apply a signal to 18А.

| COM Port |  |
| --- | --- |
| pin | purpose |
| 1 | NC |
| 2 | RXD(RS232) |
| 3 | TXD(RS232) |
| 4 | NC |
| 5 | ground |
| 6 | NC |
| 7 | NC |
| 8 | NC |
| 9 | NC |

Initialization: Open the door, enter the settings menu, close the door, choose the option ADMINISTRATION (default password is all digits 1), in this part of the menu you can change the password, create your mix of games (not more than 10) and initialize the block. In order to exit this point of menu, restart the PCB.

With the door closed at turning the key the following menu is available:

-ATTENDANT

-ACCOUNTING

-DIAGNOSTIC

-GAME HISTORY

-GAME STATICTIC

-MASHINE SETUP

With the door opened at turning the key (ground the lead 16А) additionally the following menu is available:

-ATTENDANT

-ACCOUNTING

-DIAGNOSTIC

-GAME HISTORY

-GAME STATICTIC

-MASHINE SETUP

-ADMINISTRATION